LED - Game

CONFIGURATION

Hardware:

Raspberry Pi 4 B micro controller, enclosed with heat sinks and active cooling fan

Falcon 16 channel pixel light controller with power supplies

8 strips of ws2811 bullet style pixel leds, 100 leds/strip each installed in flexible mount strap.

4 custom build controllers consisting of 1 analog joystick and 5 color lighted push buttons

Dedicated 4 port Netgear router with customized dd-wrt firmware

2 HDMI monitors, keyoard/mouse (for console)

All necessary cables for data and power.

There is a total of 8 pixel strips of ws2811 RGB bullet type LED’s. The LED strips are paired so each player controls two “Lanes” of strips.  These lanes are paralleled in 4 groups of 2. Each player controls 2 (lane left, lane right).

In summary, the entire game of four players has a total of 800 pixels.

Each of the four controllers consist of one analog joystick, a DrangonRise Inc Generic hub and five push buttons. Button 1 = red, button 2 = green, button 3 = blue, button 4 = orange, and button 5 = white.

The controllers are connected via USB powered hub to the Raspberry Pi. The current console already has the controllers mapped out.

The setup:

This build uses a Falcon 16 channel pixel controller configured for ws2811 RGB pixels. (8) 3-wire RGB pixel strings. Each string is 100 pixels (lane). Each lane is connected to a separate channel and that channel is configured as a separate ‘Universe’ in the Falcon controller. There are 8 (1-8 lanes) and are configured on the falcon controller as assigned:

Player		Lane		Channel	universe #

1		left		1		1

1		right		2		2

2		left		3		3

2		right		4		4

3		left		9		5

3		right		10		6

4		left		11		7

4		right		12		8

DMZ equipment is not currently included or configured at this time.

Input:

The pi’s input consists of keyboard & mouse (for the console), usb hub in which 4 the usb controllers are connected.

Output:

The pi’s output consists of two monitors, hdmi 0 (the players view monitor) and hdmi 1 (the display console to the host who is managing the game play). A Falcon Controller manages all the ws2811 bullet pixel strings as well as DMX in the future. Sound will be accomplished using hdmi 0 internal speak for sound effects. A dedicated router is used for communication from pi to the falcon controller.

DOT DASH

we will discuss how it’s played as if 1 player is playing. The rules will carry over to remaining players.

Game Play Operation (prestart to Dot Dash):

the console must use what's called a check-in, and the check-in on the console opens up the enrollment for each of the controllers. Once enrollment is open each player presses their white button to enroll in the game.  Once players have joined in, the host console will click Confirm Players. Those players are now locked. Once confirmed, the host has the option to play instruction slides onto the players screen to show how the game works. the host now clicks Start on the console. The leds turn off And Dot Dash will begin.

Players color selection:

On the players screen, they will be instructed to select two, different colors. For example: red and green.  As the player selects their first color, their left lane of pixels will light up corresponding to their choice. The same for their second color selection which will light up their right lane. The game will ignore any input selections after the two distinct colors have been chosen. (Joystick is also disabled.)

Once the last player had selected their two colors, the players screen will display a countdown from 3, 2, 1, and then Go! As the countdown is happening all the pixels from each play will blink red on, red off corresponding to the countdown. at the end of the countdown the lights turn green and stay on until the whoever player presses their first button- at which time the lanes turn off and an internal timer starts at 0 for all players. The players are to alternately press their previously selected color buttons as quickly as they can starting with their first color selection. As each button is correctly pushed to their respective selected colors, the left lane will advance with a single pixel at the start of their left lane. With each button pressed alternating correctly, the pixel will advance up the left pixel strip one pixel at a time with the corresponding color. If the same color is pressed two or more times in a row, their pixel “dot” will not advance forward but is recorded for metric purposes. This play continues until the end of the left lane, at which time the game play will continue down the right lane in the same alternating fashion but instead will be a cluster of 3 pixels indicating a ‘dash’. From there the dashes will advance down the right lane and finish at the bottom. Once the end has been reached; their timer stops for that player and recorded. Controller inputs for that player is ignored. Both left and right lanes of the finished player will turn on solid red for 4 seconds and then turn off. The action continues for the remaining player(s) to finish.